[image: image1.png]GUARDIAIN
HELiCOPTERS, iil .
AIR OPERATIONS




SECTION 7                                        Part 135 GH 4A   Part 133 GH 4L 
NON-Congested Area Plan

Operator:
Guardian Helicopters, Inc



16425 Hart Street



Van Nuys, CA 91406



Phone (818) 442-9904 



FAX   (818) 442-9901
PROJECT :
EDISON EXTERNAL LOAD    

WORK ORDER #: 801837213        
ADDRESS:  
ASH MOUNTAIN KAWEAH POWER HOUSE    

 
CONTRACTOR’S NAME: EDISON GENERATION      PHONE#: 760-614-0807
CONTACT PERSON: ROBERT BIEDERMANN  
ASH MOUNTAIN HELIBASE : CAREY VERNON: OFFICE (559) 565-3168 CELL  (559) 827-2009

LANDING ZONE : LATITUDE: 36° 29' 12.74"N LONGITUDE:118° 50' 23.41"W ELEVATION: 1600 FT
PICK UP : LATITUDE:  36° 27' 11.76"N LONGITUDE:118° 48' 41.44"W ELEVATION: 1300 FT
DROP  : LATITUDE: 36°28'59.24"N LONGITUDE:118°49'27.97"W ELEVATION: 2300 FT
PILOT’S NAME(S): 

1.  JOHN OLSON 


CERT #: 2769166



2. BUCKEY MACKAY
 
CERT #: 2778051



3. MARTIN RODRIGUEZ 

CERT #: 3555121



4. MICHAEL KLINK 

CERT# 33018828



TYPE OF AIRCRAFT:



BELL UH-1H
 N# N711GH
A/C CATEGORY: RESTRICTED 

BELL UH-1H
 N# N214KK
A/C CATEGORY: RESTRICTED     
BELL 407
 N# N407GH
A/C CATEGORY: STANDARD          




BELL 205A1
 N# N216GH
A/C CATEGORY: STANDARD     

AS350 B2
 N# N498PT
A/C CATEGORY: STANDARD     

AS350 B3
 N# N718GH
A/C CATEGORY: STANDARD

AS350 B3
 N# N215GH
A/C CATEGORY: STANDARD      
TYPE OF ITEM LIFTING EQUIPMENT       ( AERODYNAMIC  ( NON- AERODYNAMIC  


TYPE OF LIFTS: (20) ( < 2500 LBS CLASS OF LOAD:EXTERNAL LONG LINE:
LESS THEN 150 FT (LENGTH) OPERATIONAL ALTITUDE: < 250    STREET CLOSING:  XX
  (N0)

(YES) 

DATE AND TIMES OPERATION WILL BEGIN AND TERMINATE:

	FLIGHT DATES 
	TIME BEGIN
	TIME END
	TODAY’S DATE 

	6-17-2019 
	0800
	1600
	6-13-2019

	AGENCY NAME:
	PHONE#:
	PERSON NOTIFIED:
	DATE:

	TULARE COUNTY FIRE DEPARTMENT 
	559-561-4362
	DISPATCH 
	NOTIFIED 

	FORESTREY DEPARTMENT 
	559-561-4432
	DISPATCH
	NOTIFIED 

	ASH MOUNTAIN HELIBASE 
	559-565-3168
	CAREY VERNON 
	NOTIFIED 

	TULARE COUNTY LAKE PATROL 
	559-624-7000
	DISPATCH
	NOTIFIED 


[image: image2.png]GUARDIAIN
HELiCOPTERS, iil .
AIR OPERATIONS




NON- Congested Area Plan (Continued)
Operator:
Guardian Helicopters, Inc



16425 Hart Street



Van Nuys, CA 91406



Phone (818) 442-9904 



FAX   (818) 442-9901
NON- Congested Area Plan (Continued)

LIST OF BUILDINGS THAT SHALL EITHER BE PARTIALLY OR ENTIRELY UNOCCUPIED PERSONS ALSO HOMES THAT WILL BE NOTIFIED OF ACTIVITY:

	BUILDING DESCRIPTION/ADDRESS
	REMARKS 
	PHONE #:

	BUILDING AT DROP OFF ZONE 
	MUST BE VACANT 
	

	
	
	

	
	
	


[image: image3.jpg]50:23.35"W. «

)
z





[image: image4.png]GUARDIAIN
HELiCOPTERS, iil .
AIR OPERATIONS




NON- Congested Area Plan (Continued)

Operator:
Guardian Helicopters, Inc



16425 Hart Street



Van Nuys, CA 91406



Narrative description of pick-up site, route, delivery site, and plan for ceasing operation if unauthorized persons enter operational area or real hazard occurs.

SITE INSPECTION NOTES

ONLY PERSONNEL ESSENTIAL TO COMPLETING THE JOB SAFELY WILL BE PERMITTED TO PARTICIPATE IN THE EVOLUTION. ONLY PERSONNEL PARTICIPATING IN THE SAFETY BRIEF WILL BE PERMITTED TO BE IN THE AREA OF OPERATIONS. ONLY AUTHORIZED ESSENTIAL PERSONAL WILL BE PERMITTED IN THE IMMEDIATE AREA OF OPERATIONS. IN THE EVENT AN UNAUTHORIZED PERSON ENTERS THE OPERATIONS AREA OR A POTENTIAL HAZARD IS IDENTIFIED THE PILOT WILL HOLD THE AIRCRAFT IN RELATIVE POSITION. THIS WILL BE ACHIEVED WITHOUT OVER FLYING THE GROUND CREW. ALL OPERATIONS WILL BE CEASED UNTIL THE PROBLEM IS RECTIFIED.  IN THE EVENT OF AN AIRCRAFT EMERGENCY, THE PILOT WILL RADIO GROUND PERSONNEL AND WILL ATTEMPT TO LAND IN THE EMERGENCY LANDING AREA. 

DURING THE LIFT EVOLUTION THE AIRCRAFT AND GROUND CREW WILL ALL BE IN RADIO COMMUNICATIONS. IN THE EVENT OF LOSS COMMUNICATIONS THE CREW WILL USE STANDARD HAND SIGNALS TO COMMUNICATE WITH THE PILOT. WHEN THE PILOT IS CLEAR HE WILL MOVE THE AIRCRAFT TO A SAFE LOCATION TO ALLOW THE COMMUNICATION PROBLEM TO BE RECTIFIED.   

THE OPERATIONS AREA IS WITHIN THE SECURED AREA. ALL ESSENTIAL CREW WITHIN THE OPERATIONS AREA WILL NOT BE INVOLVED IN TAKING PICTURES OR ANY OTHER USE OF CELL PHONES.  

1. HELICOPTER WILL LAND AT THE HELIPAD TO CONNECTING LONGLINE. THE AIRCRAFT WILL FLY THE EMPTY LONGLINE FROM THE HELIPORT TO THE PICK UP AREA CROSSING THE ROAD WHEN CLEAR. 
2. TAILBOARD AT LANDING ZONE ASH MOUNTAIN HELIBASE AT 0730 
3.  EMERGENCY LANDING ZONE IS AT THE HELIPAD, STAGING AREA AND IN ROUTE TO PICK UP AREA 
4. WATER TRUCK IS NOT REQUIRED  

5. BUILDING NEAR DROP OFF ZONE MUST BE VACANT 

6. FLAGMAN BASED AT BRIDGE ON THE ROAD TO DROP OFF ZONE 

6. AIRCRAFT WILL FUEL AT HELIBASE. 
7. CUSTOMER WILL TAKE RESPONSIBILITY TO RIG ITEMS BEING LIFTED. 

[image: image5.jpg]159.24N 118°49:27-97" W

P
Al

Google Earth-e-s; -




[image: image6.jpg]elevi1641ft  eyealt 2658ft O




[image: image7.jpg]



[image: image8.jpg]



[image: image9.jpg]= B
36°29/07.047 N 118°50107.49" W elev 1374 ft eye alt 2239 ft )





[image: image10.jpg]



LEGEND

	SITE PLAN SYMBOLS: EVACUATION FLIGHT PATH

	LANDING AREA = (LZ)
	STREET BLOCKED = XXXX
	TOP TWO FLOORS = UBT2

	ATTACH AREA = (A)
	RELEASE AREA = (R)
	

	FLIGHT PATH = >>>
	EMERGENCY LAND = (ELZ)
	

	UNOCCUPIED BUILDING = (UBA) 
	UNOCCUPIED BUILDING TOP FLOOR = (UBT1)
	


6-13-2019



[image: image11.jpg]








DATE OF SUBMISSION



COMPANY OFFICIAL / TITLE

