[image: image1.png]GUARDIAIN
HELiCOPTERS, iil .
AIR OPERATIONS




SECTION 7                                        Part 135 GH 4A   Part 133 GH 4L 
NON-Congested Area Plan                                                       
Operator:
Guardian Helicopters, Inc



16425 Hart Street



Van Nuys, CA 91406



Phone (818) 442-9904 



FAX   (818) 442-9901
PROJECT: EDISON POLE REMOVAL WORK ORDER NUMBER: TD 1269171 TD 1296352 TD 1425666 TD 1408703 
ADDRESS:  
CHINA PEAK / KAISER PASS RD  :SHAVER LAKE CA   

 
LZ STAGING AREA LATTITUDE: 37°14' 8.14 "N LONGITUDE 119°9' 32.97"W
POLE STAGING AREA LATTITUDE: 37°16' 46.66 "N LONGITUDE 119°6' 32.15"W
POLE 1807033E LATTITUDE: 37°16' 52.92 "N LONGITUDE 119°6' 35.51"W ELEV 8463 FT 1300 LBS
POLE STAGING AREA: LATTITUDE: 37°19'27.54"N LONGITUDE 119° 5'12.37"W
POLE 1722698E: LATTITUDE: 37°19'02.01"N LONGITUDE 119° 4' 31.29"W ELEV 7400 FT 2000 LBS
 POLE 1722686E: LATTITUDE: 37°18' 46.53"N LONGITUDE 119° 5' 2.15"W ELEV 7800 FT 2000 LBS
POLE 1722667E: LATTITUDE: 37°18' 08.18"N LONGITUDE 119° 5' 35.04"W ELEV 9078 FT1300 LBS 
CONTRACTOR’S NAME: SCE T&D CHARLES CALVO      PHONE#: 559-280-7850

CONTACT PERSON: COBY WHITE HOTLINE PHONE#: 925-529-0524
PILOT’S NAME(S): 
1.  JOHN OLSON 


CERT #: 2769166



2. BUCKEY MACKAY
 
CERT #: #:2778051



3. MARTIN RODRIGUEZ 

CERT #: 3555121



4. MICHAEL KLINK 

CERT# 33018828


TYPE OF AIRCRAFT:



BELL UH-1H
 N# N711GH
A/C CATEGORY: RESTRICTED 

BELL UH-1H
 N# N214KK
A/C CATEGORY: RESTRICTED 

BELL 407
 N# N407GH
A/C CATEGORY: STANDARD          

 


BELL 205A1
 N# N216GH
A/C CATEGORY: STANDARD     

AS350 B2
 N# N498PT
A/C CATEGORY: STANDARD     

AS350 B3
 N# N718GH
A/C CATEGORY: STANDARD

AS350 B3
 N# N215GH
A/C CATEGORY: STANDARD      
TYPE OF ITEM LIFTING: POLE      ( AERODYNAMIC  ( NON- AERODYNAMIC  


IF SAFETY DICTATES AND OR THE CUSTOMER REQUESTS ADDITIONAL LIFTS THE LIFTS WILL BE CONDUCTED REMAINING WITHIN THE APPROVED TIME PARAMETERS. 
TYPE OF LIFTS: (4) MAX 2000 LBS     
MAX SINGLE – LIFT WEIGHT:
2000   LBS
CLASS OF LOAD:
EXTERNAL CL B  


NO. OF LIFTS:  8
LONG LINE:
 
(NO)
XXX
(YES)
LESS THEN 150 FT 
(LENGTH)

OPERATIONAL ALTITUDE: < 50    STREET CLOSING:  XX
  (N0)

(YES) 

DATE AND TIMES OPERATION WILL BEGIN AND TERMINATE:

	FLIGHT DATES 
	TIME BEGIN
	TIME END
	TODAY’S DATE 

	7-29-2019
	SUNRISE 
	SUNSET 
	7-25-2019

	AGENCY NAME:
	PHONE#:
	PERSON NOTIFIED:
	DATE:

	CHP  DEPARTMENT
	559-841-8473
	DISPATCH 
	NOTIFY 

	FIRE DEPARTMENT 
	559-841-3360
	DISPATCH  
	NOTIFY

	CAL FIRE 
	559-841-7774
	DISPATCH 
	NOTIFY 


[image: image2.png]GUARDIAIN
HELiCOPTERS, iil .
AIR OPERATIONS




NON- Congested Area Plan (Continued)
Operator:
Guardian Helicopters, Inc



16425 Hart Street



Van Nuys, CA 91406



Phone (818) 442-9904 



FAX   (818) 442-9901
LIST OF BUILDINGS THAT SHALL EITHER BE PARTIALLY OR ENTIRELY UNOCCUPIED PERSONS ALSO HOMES THAT WILL BE NOTIFIED OF ACTIVITY:

	BUILDING DESCRIPTION/ADDRESS
	REMARKS 
	PHONE #:

	NONE
	
	

	
	
	

	
	
	


[image: image3.jpg]1722698E 372192:01'N 1

752686





[image: image4.png]GUARDIAIN
HELiCOPTERS, iil .
AIR OPERATIONS




NON- Congested Area Plan (Continued)

Operator:
Guardian Helicopters, Inc



16425 Hart Street



Van Nuys, CA 91406



Narrative description of pick-up site, route, delivery site, and plan for ceasing operation if unauthorized persons enter operational area or real hazard occurs.

SITE INSPECTION NOTES

ONLY PERSONNEL ESSENTIAL TO COMPLETING THE JOB SAFELY WILL BE PERMITTED TO PARTICIPATE IN THE EVOLUTION. ONLY PERSONNEL PARTICIPATING IN THE SAFETY BRIEF WILL BE PERMITTED TO BE IN THE AREA OF OPERATIONS. ONLY AUTHORIZED ESSENTIAL PERSONAL WILL BE PERMITTED IN THE IMMEDIATE AREA OF OPERATIONS. IN THE EVENT AN UNAUTHORIZED PERSON ENTERS THE OPERATIONS AREA OR A POTENTIAL HAZARD IS IDENTIFIED THE PILOT WILL HOLD THE AIRCRAFT IN RELATIVE POSITION. THIS WILL BE ACHIEVED WITHOUT OVER FLYING THE GROUND CREW. ALL OPERATIONS WILL BE CEASED UNTIL THE PROBLEM IS RECTIFIED.  IN THE EVENT OF AN AIRCRAFT EMERGENCY, THE PILOT WILL RADIO GROUND PERSONNEL AND WILL ATTEMPT TO LAND IN THE EMERGENCY LANDING AREA. 

DURING THE LIFT EVOLUTION THE AIRCRAFT AND GROUND CREW WILL ALL BE IN RADIO COMMUNICATIONS. IN THE EVENT OF LOSS COMMUNICATIONS THE CREW WILL USE STANDARD HAND SIGNALS TO COMMUNICATE WITH THE PILOT. WHEN THE PILOT IS CLEAR HE WILL MOVE THE AIRCRAFT TO A SAFE LOCATION TO ALLOW THE COMMUNICATION PROBLEM TO BE RECTIFIED.   

THE OPERATIONS AREA IS WITHIN THE SECURED AREA. ALL ESSENTIAL CREW WITHIN THE OPERATIONS AREA WILL NOT BE INVOLVED IN TAKING PICTURES OR ANY OTHER USE OF CELL PHONES.  

A DAILY TAILBOARD SAFETY MEETING WILL BE HELD AT THE ASSIGNED LOCATING PRIOR TO BEING OPERATIONS 

1. HELICOPTER WILL LAND AT THE STAGING AREA  FOR FUELING AND CONNECTING LONGLINE 
2. POLES WILL BE STAGED AT ASSIGNED POLE STAGING AREAS  
3. WATER TRUCK NOT REQUIRED  
4. EMERGENCY LANDING ZONE IS AT THE LANDING ZONE , STAGING AREA AND IN ROUTE TO POLE SITES IF REQUIRED 
5. FLAG MAN WILL BE POSTED TO STOP ANY TRAFFIC WHILE THE HELICOPTER IS SETTING POLES NEAR THE ROAD. SEE DIAGRAM   
6. ROAD LEADING TO LZ STAGING AREA WILL BE CLOSED.   

[image: image5.jpg]



[image: image6.jpg]e






[image: image7.jpg]


[image: image8.jpg]


LEGEND

	SITE PLAN SYMBOLS: EVACUATION FLIGHT PATH

	LANDING AREA = (LZ)
	STREET BLOCKED = XXXX
	TOP TWO FLOORS = UBT2

	ATTACH AREA = (A)
	RELEASE AREA = (R)
	

	FLIGHT PATH = >>>
	EMERGENCY LAND = (ELZ)
	

	UNOCCUPIED BUILDING = (UBA) 
	UNOCCUPIED BUILDING TOP FLOOR = (UBT1)
	


7-25-2019




[image: image9.jpg]








DATE OF SUBMISSION



COMPANY OFFICIAL / TITLE

