SECTION 7                                        Part 135 GH 4A   Part 133 GH 4L

NON-Congested Area Plan

Operator:	Guardian Helicopters, Inc

67 D STREET

Fillmore CA. 93015

Phone (818) 442-9904

FAX   (818) 442-9901

PROJECT: 	EDISON POLE REPLACEMENT   	WORK ORDER #: TD 1539636

ADDRESS:  TABLE MOUNTAIN RD  	:  WRIGHTWOOD         CA

LZ STAGING AREA  LATITUDE 34°22'52.73"N LONGITUDE 117°41'25.70"W ELV 7300 FT

1001150E   LATITUDE 34°22'54.92"N LONGITUDE 117°41'16.79"W 40CL2 1300 LBS

CONTRACTOR’S NAME: DIVERSIFIED          	 PHONE#: 661-437-2819

PILOT’S NAME(S):

1.  JOHN OLSON 			CERT #: 2769166

2. BUCKEY MACAY	 	CERT #: 2778051

3. MARTIN RODRIGUEZ 		CERT #: 3555121

4. MICHAEL KLINK 		CERT# 33018828

5. IAN GOODALE			CERT# 3741973

TYPE OF AIRCRAFT:

BELL UH-1H	 N# N711GH	A/C CATEGORY: RESTRICTED

BELL UH-1H	 N# N214KK	A/C CATEGORY: RESTRICTED

BELL 407	 N# N407GH	A/C CATEGORY: STANDARD

BELL 205A1	 N# N216GH	A/C CATEGORY: STANDARD

AS350 B2	 N# N498PT	A/C CATEGORY: STANDARD

AS350 B3	 N# N718GH	A/C CATEGORY: STANDARD

AS350 B3	 N# N215GH	A/C CATEGORY: STANDARD

TYPE OF ITEM LIFTING: POLE       AERODYNAMIC   NON- AERODYNAMIC

IF SAFETY DICTATES AND OR THE CUSTOMER REQUESTS ADDITIONAL LIFTS THE LIFTS WILL BE CONDUCTED REMAINING WITHIN THE APPROVED TIME PARAMETERS.

TYPE OF LIFTS: (1)  (1) < POLES LESS 1400 LBS LBS, EQUIPMENT

MAX SINGLE – LIFT WEIGHT:	1400  LBS	CLASS OF LOAD:	EXTERNAL

NO. OF LIFTS:  TBD	LONG LINE:	 	(NO)	XXX	(YES)	LESS THEN 150 FT 	(LENGTH)

OPERATIONAL ALTITUDE: < 250 FT    STREET CLOSING:  XX	  (N0)		(YES)

DATE AND TIMES OPERATION WILL BEGIN AND TERMINATE:

| FLIGHT DATES | TIME BEGIN | TIME END | TODAY’S DATE |
| --- | --- | --- | --- |
| 10-31-2019 | 0800 | 1600 | 10-25-2019 |
| AGENCY NAME: | PHONE#: | PERSON NOTIFIED: | DATE: |
| POLICE DEPARTMENT | 661-272-2400 | WATCH COMMANDER | WILL NOTIFY |
| FIRE DEPARTMENT | 661-944-1260 | DISPATCH | WILL NOTIFY |
| ANSF | EMAIL |  | SENT |

SECTION 7                                        Part 135 GH 4A   Part 133 GH 4L

NON-Congested Area Plan

LIST OF BUILDINGS THAT SHALL EITHER BE PARTIALLY OR ENTIRELY UNOCCUPIED PERSONS:

| BUILDING DESCRIPTION/ADDRESS | OWNER(S): | PHONE #: |
| --- | --- | --- |
| N/A | N/A | N/A |

Evacuation 					Crowd Control 			Street Closure

No Evacuated Structures 			Operator 			Yes □ No

□Evacuated Structures 				Contractor			Contractor

□Operator

Narrative description of pick-up site, route, delivery site, and plan for ceasing operation if unauthorized persons enter operational area or real hazard occurs.

SITE INSPECTION NOTES

ONLY PERSONNEL ESSENTIAL TO COMPLETING THE JOB SAFELY WILL BE PERMITTED TO PARTICIPATE IN THE EVOLUTION. ONLY PERSONNEL PARTICIPATING IN THE SAFETY BRIEF WILL BE PERMITTED TO BE IN THE AREA OF OPERATIONS. ONLY AUTHORIZED ESSENTIAL PERSONAL WILL BE PERMITTED IN THE IMMEDIATE AREA OF OPERATIONS. IN THE EVENT AN UNAUTHORIZED PERSON ENTERS THE OPERATIONS AREA OR A POTENTIAL HAZARD IS IDENTIFIED THE PILOT WILL HOLD THE AIRCRAFT IN RELATIVE POSITION. THIS WILL BE ACHIEVED WITHOUT OVER FLYING THE GROUND CREW. ALL OPERATIONS WILL BE CEASED UNTIL THE PROBLEM IS RECTIFIED.  IN THE EVENT OF AN AIRCRAFT EMERGENCY, THE PILOT WILL RADIO GROUND PERSONNEL AND WILL ATTEMPT TO LAND IN THE EMERGENCY LANDING AREA. THE EMERGENCY LANDING AREA IS IN THE FIELD TO THE WEST OF FENNNER CAMP #41.  THE HELICOPTER WILL LAND IN THE LZ TO HOOK UP THE LONG LINE.  REMOTE AREA NO BUILDINGS IN OPERATING AREA .

DURING THE LIFT EVOLUTION THE AIRCRAFT AND GROUND CREW WILL ALL BE IN RADIO COMMUNICATIONS. IN THE EVENT OF LOSS COMMUNICATIONS THE CREW WILL USE STANDARD HAND SIGNALS TO COMMUNICATE WITH THE PILOT. WHEN THE PILOT IS CLEAR HE WILL MOVE THE AIRCRAFT TO A SAFE LOCATION TO ALLOW THE COMMUNICATION PROBLEM TO BE RECTIFIED.

THE OPERATIONS AREA IS WITHIN THE SECURED AREA. ALL ESSENTIAL CREW WITHIN THE OPERATIONS AREA WILL NOT BE INVOLVED IN TAKING PICTURES OR ANY OTHER USE OF CELL PHONES.

A SAFETY BRIEF TAILBOARD WILL BE CONDUCTED AT THE STAGING AREA PRIOR TO WORK EVERYDAY.

1. LANDING ZONE AND EMERGENCY LANDING ZONE IS OFF OF TABLE MOUNTAIN RD

2. TABLE MOUNTAIN RD  WILL BE CLOSED INTERMITTENTLY DURING OPERATION

3. ALL AREA IN THE SECURE AREA WILL BE UNOCCUPIED.

4. PEDESTRIANS WILL NOT BE PERMITTED OPERATING AREA

5. RESIDENTIAL EVACUATION IN NOT REQUIRED

6. WATER TRUCK IS REQUIRED AT LZ STAGING AREA

7. SECURITY/ STREET CLOSURE WILL BE PROVIDED SEE PLAN

8. A SPECIAL USAGE NOTIFICATION HAS BEEN SUBMITTED TO THE ANGELS FOREST SERVICE

9. CUSTOMER WILL ENSURE TREE TRIMMING HAS OCCURRED IF REQUIRED.

10-25-2019

DATE OF SUBMISSION				COMPANY OFFICIAL / TITLE

OPERATIONS AREA DIAGRAM

LEGEND

| SITE PLAN SYMBOLS: EVACUATION FLIGHT PATH |  |  |
| --- | --- | --- |
| LANDING AREA = (LZ) | STREET BLOCKED = XXXX | TOP TWO FLOORS = UBT2 |
| ATTACH AREA = (A) | RELEASE AREA = (R) |  |
| FLIGHT PATH = >>> | EMERGENCY LAND = (ELZ) |  |
| UNOCCUPIED BUILDING = (UBA) | UNOCCUPIED BUILDING TOP FLOOR = (UBT1) |  |

AREA SURVEY CONDUCTED BY :