[image: image1.png]GUARDIAIN
HELiCOPTERS, iil .
AIR OPERATIONS




SECTION 7                                        Part 135 GH 4A   Part 133 GH 4L 
NON-Congested Area Plan

Operator:
Guardian Helicopters, Inc



67 D STREET 


Fillmore CA. 93015 



Phone (818) 442-9904 



FAX   (818) 442-9901
PROJECT: 
EDISON POLE REPLACEMENT   
WORK ORDER #: TD 1479005
ADDRESS:  DEEP CREEK CAMP ROAD      
:  CRESTLINE   CA
 

LZ STAGING AREA  LATITUDE 34°15'1.30"N LONGITUDE 117° 8'13.20"W ELEV 5500 FT
POLE 4903057E  LATITUDE 34°14'37.47"N LONGITUDE 117° 8'24.87"W 45 H2 WOOD 2400 LBS
POLE 4903058E  LATITUDE 34°14'35.73"N LONGITUDE 117° 8'24.15"W 45 CL 1 COMP1200 LBS
POLE 4903059E  LATITUDE 34°14'34.48"N LONGITUDE 117° 8'23.45"W 45 H1 WOOD  2200 LBS
POLE 4903060E  LATITUDE 34°14'32.67"N LONGITUDE 117° 8'22.62"W H2 WOOD 2400 LBS
POLE 4903061E  LATITUDE 34°14'29.01"N LONGITUDE 117° 8'20.84"W45 H1 WOOD 2200 LBS
POLE 4903055E LATITUDE 34°14'27.45"N LONGITUDE 117° 8'20.12"W 45 CL 1 COMP1200 LBS
POLE 4903056E  LATITUDE 34°14'24.02"N LONGITUDE 117° 8'18.45"W 45 CL 1 COMP1200 LBS
CONTRACTOR’S NAME: GREG OWENS      
  
  PHONE#:  310-968-6148

CONTACT AT SITE;   GABRIEL TREVINO

 PHONE#: 714-788-0864
PILOT’S NAME(S): 
1.  JOHN OLSON 


CERT #: 2769166



2. BUCKEY MACKAY
 
CERT #: #:2778051



3. MARTIN RODRIGUEZ 

CERT #: 3555121



4. MICHAEL KLINK 

CERT# 33018828



TYPE OF AIRCRAFT:BELL UH-1H
 N# N711GH
A/C CATEGORY: RESTRICTED 

BELL UH-1H
 N# N214KK
A/C CATEGORY: RESTRICTED 

BELL 407
 N# N407GH
A/C CATEGORY: STANDARD          




BELL 205A1
 N# N216GH
A/C CATEGORY: STANDARD     

AS350 B2
 N# N498PT
A/C CATEGORY: STANDARD     

AS350 B3
 N# N718GH
A/C CATEGORY: STANDARD

AS350 B3
 N# N215GH
A/C CATEGORY: STANDARD      
TYPE OF ITEM LIFTING: POLE      ( AERODYNAMIC  ( NON- AERODYNAMIC  


IF SAFETY DICTATES AND OR THE CUSTOMER REQUESTS ADDITIONAL LIFTS THE LIFTS WILL BE CONDUCTED REMAINING WITHIN THE APPROVED TIME PARAMETERS. 
TYPE OF LIFTS: (1) 50 FT CL 2   2500 LS (1) 45 CL 1 2200 BLS CLASS OF LOAD:EXTERNAL CL B  


MAX 2500 LBS NO. OF LIFTS:  2
LONG LINE:
 (NO)
XXX
(YES)
LESS THEN 150 FT (LENGTH)

OPERATIONAL ALTITUDE: < 250    STREET CLOSING:  
  (N0)
XX
(YES) 

DATE AND TIMES OPERATION WILL BEGIN AND TERMINATE:

	FLIGHT DATES 
	TIME BEGIN
	TIME END
	TODAY’S DATE 

	10-8-2019
	SUNRISE 
	SUNSET 
	8-23-2019

	AGENCY NAME:
	PHONE#:
	PERSON NOTIFIED:
	DATE:

	TWIN PEAKS  POLICE
	909-336-0600
	WATCH COMMANDER
	NOTIFY 

	LA COUNTY FIRE
	626-797-0379
	STATION 11
	NOTIFY


[image: image2.png]GUARDIAIN
HELiCOPTERS, iil .
AIR OPERATIONS




SECTION 7                                        Part 135 GH 4A   Part 133 GH 4L 
NON-Congested Area Plan

Operator:
Guardian Helicopters, Inc



67 D Street 



Fillmore, California 93015



Evacuation 




Crowd Control 


Street Closure 

[image: image3.jpg]


No Evacuated Structures 


[image: image4.jpg]


Operator 



[image: image5.jpg]


Yes □ No 

□Evacuated Structures 


[image: image6.jpg]


Contractor



[image: image7.jpg]


Contractor












□Operator 

LIST OF BUILDINGS THAT SHALL EITHER BE PARTIALLY OR ENTIRELY UNOCCUPIED PERSONS ALSO HOMES THAT WILL BE NOTIFIED OF ACTIVITY:

	BUILDING DESCRIPTION/ADDRESS
	REMARKS 
	METHOD 

	NONE     
	VACANT DURING OPS 
	HAND DELIVER 

	ALL HOMES WITHIN OPERATING AREA 
	VACANT DURING OPS 
	HAND DELIVER 

	SEE DIAGRAM = V 
	VACANT DURING OPS 
	HAND DELIVER 


Narrative description of pick-up site, route, delivery site, and plan for ceasing operation if unauthorized persons enter operational area or real hazard occurs.

SITE INSPECTION NOTES

ONLY PERSONNEL ESSENTIAL TO COMPLETING THE JOB SAFELY WILL BE PERMITTED TO PARTICIPATE IN THE EVOLUTION. ONLY PERSONNEL PARTICIPATING IN THE SAFETY BRIEF WILL BE PERMITTED TO BE IN THE AREA OF OPERATIONS. ONLY AUTHORIZED ESSENTIAL PERSONAL WILL BE PERMITTED IN THE IMMEDIATE AREA OF OPERATIONS. IN THE EVENT AN UNAUTHORIZED PERSON ENTERS THE OPERATIONS AREA OR A POTENTIAL HAZARD IS IDENTIFIED THE PILOT WILL HOLD THE AIRCRAFT IN RELATIVE POSITION. THIS WILL BE ACHIEVED WITHOUT OVER FLYING THE GROUND CREW. ALL OPERATIONS WILL BE CEASED UNTIL THE PROBLEM IS RECTIFIED.  IN THE EVENT OF AN AIRCRAFT EMERGENCY, THE PILOT WILL RADIO GROUND PERSONNEL AND WILL ATTEMPT TO LAND IN THE EMERGENCY LANDING AREA. 

DURING THE LIFT EVOLUTION THE AIRCRAFT AND GROUND CREW WILL ALL BE IN RADIO COMMUNICATIONS. IN THE EVENT OF LOSS COMMUNICATIONS, THE CREW WILL USE STANDARD HAND SIGNALS TO COMMUNICATE WITH THE PILOT. WHEN THE PILOT IS CLEAR HE WILL MOVE THE AIRCRAFT TO A SAFE LOCATION TO ALLOW THE COMMUNICATION PROBLEM TO BE RECTIFIED.   

THE OPERATIONS AREA IS WITHIN THE SECURED AREA. ALL ESSENTIAL CREW WITHIN THE OPERATIONS AREA WILL NOT BE INVOLVED IN TAKING PICTURES OR ANY OTHER USE OF CELL PHONES.  

1. HELICOPTER WILL LAND IN THE LANDING ZONE TO CONDUCT A TAILBOARD AND FOR ALL FUELING OPERATIONS. 

2. PRIOR TO COMMENCING OPERATIONS PILOT WILL FLY A RECON FLIGHT   

3. EMERGENCY LANDING ZONE IS AT THE STAGING AREA AND ALONG THE FLIGHT PATH 

4 NO HOMES OR BUILDINGS WILL REQUIRE EVACAUTION 
5. WATER TRUCK NOT REQUIRED  

6. FLAGMAN WILL STOP TRAFFIC WILL BE PROVIDED NEAR THE LZ SEE DIAGRAM 
7. POLES WILL BE STAGED AT THE STAGING AREA   
8. PLEASE TRIM TREES IF REQUIRED
[image: image8.jpg]4903060E 34°14:
49050565 344





[image: image9.jpg]©2015 Googe.




[image: image10.jpg]



[image: image11.jpg]



10-1-2019



[image: image12.jpg]








DATE OF SUBMISSION



COMPANY OFFICIAL / TITLE

