SECTION 7                                        Part 135 GH 4A   Part 133 GH 4L

NON Congested Area Plan

Operator:	Guardian Helicopters, Inc

67 D STREET 					Phone (818) 442-9904

Fillmore CA. 93015 					FAX   (818) 442-9901

PROJECT: EDISON HEC AND POLE REPLACEMENT  WORK ORDER #: TD 1394479

ADDRESS DAVEPORT ROAD           	:  SANTA CLARITA            CA

LANDING ZONE LATITUDE 34°28'33.73"N LONGITUDE 118°21'3.19"W

4921417E LATITUDE 34°28'36.43"N LONGITUDE 118°21'9.45"W 45 CL 3 1300 LBS

CONTACT AT SITE:	PEDRO LEMUS PHONE#: 909-306-8203

CONTRACTOR:    HAMPTON TEDDER       PHONE#: 909-306-8203

PILOT’S NAME(S): 	1.  JOHN OLSON 			CERT #: 2769166

2. BUCKEY MACKAY	 	CERT #: 2778051

3. MICHAEL KLINK 		CERT# 3018828

4. IAN GOODALE			CERT# 3741973

TYPE OF AIRCRAFT:BELL UH-1H	 N# N711GH	A/C CATEGORY: RESTRICTED

BELL UH-1H	 N# N214KK	A/C CATEGORY: RESTRICTED

BELL 407	 N# N407GH	A/C CATEGORY: STANDARD

BELL 205A1	 N# N216GH	A/C CATEGORY: STANDARD

AS350 B2	 N# N498PT	A/C CATEGORY: STANDARD

AS350 B3	 N# N718GH	A/C CATEGORY: STANDARD

AS350 B3	 N# N215GH	A/C CATEGORY: STANDARD

IF SAFETY DICTATES AND OR THE CUSTOMER REQUESTS ADDITIONAL LIFTS THE LIFTS WILL BE CONDUCTED REMAINING WITHIN THE APPROVED TIME PARAMETERS.

TYPE OF LIFTS: (3 ) POLES  10 LOADS HEAVIEST 2400 LBS COMPRESSOR   LOAD:EXTL CL B  TYPE OF ITEM LIFTING:  AERODYNAMIC   NON- AERODYNAMIC  	 MAX  2700 LBS

NO. OF LIFTS: 10	LONG LINE:	 	(NO)	XXX	(YES)	LESS THEN 150 FT 	(LENGTH)

OPERATIONAL ALTITUDE: < 250    STREET CLOSING:  	  (N0)	XX	(YES)

DATE AND TIMES OPERATION WILL BEGIN AND TERMINATE:

| FLIGHT DATES | TIME BEGIN | TIME END | TODAY’S DATE |
| --- | --- | --- | --- |
| 2-12-2020 | SUNRISE | SUNSET | 2-6-2020 |
| AGENCY NAME: | PHONE#: | PERSON NOTIFIED: | DATE: |
| POLICE DEPARTMENT | 661-272-2400 | WATCH COMMANDER | WILL NOTIFY |
| FIRE DEPARTMENT | 661-944-1260 | DISPATCH | WILL NOTIFY |

Evacuation 					Crowd Control 			Street Closure

No Evacuated Structures 		 		Operator 			Yes  □No

□Evacuated Structures 	      			 Contractor		Contractor

□Operator

Part 135 GH 4A   Part 133 GH 4L

NON Congested Area Plan

LIST OF BUILDINGS THAT SHALL EITHER BE PARTIALLY OR ENTIRELY UNOCCUPIED PERSONS ALSO HOMES THAT WILL BE NOTIFIED OF ACTIVITY:

| BUILDING DESCRIPTION/ADDRESS | REMARKS | METHOD |
| --- | --- | --- |
| REMOTE NO EVACAUTIONS | VACANT DURING OPS | HAND DELIVER |
| ALL HOMES WITHIN RED OPERATING AREA | VACANT DURING OPS | HAND DELIVER |
| SEE DIAGRAM = VACANT | VACANT DURING OPS | HAND DELIVER |

Narrative description of pick-up site, route, delivery site, and plan for ceasing operation if unauthorized persons enter operational area or real hazard occurs.

SITE INSPECTION NOTES

ONLY PERSONNEL ESSENTIAL TO COMPLETING THE JOB SAFELY WILL BE PERMITTED TO PARTICIPATE IN THE EVOLUTION. ONLY PERSONNEL PARTICIPATING IN THE SAFETY BRIEF WILL BE PERMITTED TO BE IN THE AREA OF OPERATIONS. ONLY AUTHORIZED ESSENTIAL PERSONAL WILL BE PERMITTED IN THE IMMEDIATE AREA OF OPERATIONS. IN THE EVENT AN UNAUTHORIZED PERSON ENTERS THE OPERATIONS AREA OR A POTENTIAL HAZARD IS IDENTIFIED THE PILOT WILL HOLD THE AIRCRAFT IN RELATIVE POSITION. THIS WILL BE ACHIEVED WITHOUT OVER FLYING THE GROUND CREW. ALL OPERATIONS WILL BE CEASED UNTIL THE PROBLEM IS RECTIFIED.  IN THE EVENT OF AN AIRCRAFT EMERGENCY, THE PILOT WILL RADIO GROUND PERSONNEL AND WILL ATTEMPT TO LAND IN THE EMERGENCY LANDING AREA.

DURING THE LIFT EVOLUTION THE AIRCRAFT AND GROUND CREW WILL ALL BE IN RADIO COMMUNICATIONS. IN THE EVENT OF LOSS COMMUNICATIONS, THE CREW WILL USE STANDARD HAND SIGNALS TO COMMUNICATE WITH THE PILOT. WHEN THE PILOT IS CLEAR HE WILL MOVE THE AIRCRAFT TO A SAFE LOCATION TO ALLOW THE COMMUNICATION PROBLEM TO BE RECTIFIED.

THE OPERATIONS AREA IS WITHIN THE SECURED AREA. ALL ESSENTIAL CREW WITHIN THE OPERATIONS AREA WILL NOT BE INVOLVED IN TAKING PICTURES OR ANY OTHER USE OF CELL PHONES.

PRIOR TO BEGINNING OPERATIONS, A TAILBOARD SAFETY BRIEF WILL BE CONDUCTED.

- THE HELICOPTER WILL LAND AT THE LZ TO CONNECT THE LONGLINE AND CONDUCT A SAFETY TAILBOARD

- WATER TRUCK IS REQUIRED AT THE LANDING ZONE

- POLE AND EQUIPMENT WILL BE STAGED AT THE LANDING ZONE

- FUELING WILL BE CONDUCTED AT THE LANDING ZONE

- EMERGENCY LANDING ZONE IS AT THE STAGING AREA AND ALONG THE FLIGHT PATH

- RADIOS WILL BE PROVIDED TO EACH POLE SITE

- DAVENPORT WILL EXPERIENCE INTERMITTENT CLOSURES

- REMOTE NO EVACUATIONS

- THE HELCOPTER WILL CONDUCT A LOW RECON FLIGHT PRIOR TO STARTING OPERATIONS. THE HELICOPTER WILL LAND AT THE LZ TO CONDUCT A SAFETY TAILBOARD AND WHEN THE CREW IS READY TO SET THE POLE DAVENPORT WILL BE CLOSED DOWN. ONE THE POLE IS SET AND THE OLD POLE IS REMOVED DAVENPORT WILL BE REOPENED. THE HELICOPTER WILL LAND AT THE LZ TO WAIT TO REMOVE TOOLS AND EQUIPMENT. AFTER REMOVAL OF THE TOOLS AND EQUIPMENT THE AIRCRAFT WILL DEPART THE AREA.

- THE CONTRACTOR WILL TRIM TREES IF REQUIRED

2-3-2020

DATE OF SUBMISSION				COMPANY OFFICIAL / TITLE