SECTION 7                                        Part 135 GH 4A   Part 133 GH 4L

Congested Area Plan

Operator:	Guardian Helicopters, Inc

67 D STREET 							Phone (818) 442-9904

Fillmore CA. 93015 							FAX   (818) 442-9901

PROJECT: 	EDISON LINE PULL    	WORK ORDER #: TD 1504506

ADDRESS:  CLARK RANCH ROAD AND CREEK RD               	:  OJIA      CA

LZ STATING AREA LATITUDE 34°24'32.62"N LONGITUDE 119°16'34.83"W

START POINT LATITUDE 34°24'34.16"N LONGITUDE 119°16'33.03"W

STOP POINT  LATITUDE 34°24'29.10"N LONGITUDE 119°15'53.50"W

COMMUNITY MEMORIAL HOSPITAL VENTURA  34 16 29 – 119 15 25 FAA LOC ID CA72 02401.51*H

CONTACT AT SITE;   JAROD GARCIA            	     PHONE#: 209-470-6069

PILOT’S NAME(S): 	1.  JOHN OLSON 			CERT #: 2769166

2. BUCKEY MACKAY	 	CERT #: 2778051

3. MICHAEL KLINK 		CERT# 3018828

4. IAN GOODALE			CERT# 3741973

5. ROSS GOULET 			CERT# 339677

TYPE OF AIRCRAFT:

BELL UH-1H	 N# N711GH	A/C CATEGORY: RESTRICTED

BELL UH-1H	 N# N777GH	A/C CATEGORY: RESTRICTED

BELL 407	 N# N407GH	A/C CATEGORY: STANDARD

BELL 205A1	 N# N216GH	A/C CATEGORY: STANDARD

AS350 B2	 N# N498PT	A/C CATEGORY: STANDARD

AS350 B3	 N# N718GH	A/C CATEGORY: STANDARD

AS350 B3	 N# N215GH	A/C CATEGORY: STANDARD

IF SAFETY DICTATES AND OR THE CUSTOMER REQUESTS ADDITIONAL LIFTS THE LIFTS WILL BE CONDUCTED REMAINING WITHIN THE APPROVED TIME PARAMETERS.

TYPE OF LIFTS: LINE PULL      CLASS OF LOAD:EXTL CL B  TYPE OF ITEM LIFTING:   ERODYNAMIC   NON- AERODYNAMIC  	 MAX  1500 LBS

NO. OF LIFTS: TBD	LONG LINE:	 	(NO)	XXX	(YES)	LESS THEN 150 FT	(LENGTH)

OPERATIONAL ALTITUDE: < 250    STREET CLOSING:  	  (N0)	XX	(YES)

DATE AND TIMES OPERATION WILL BEGIN AND TERMINATE:

| FLIGHT DATES | TIME BEGIN | TIME END | TODAY’S DATE |
| --- | --- | --- | --- |
| 4-13-2020 | SUNRISE | SUNSET | 4-9-2020 |
| AGENCY NAME: | PHONE#: | PERSON NOTIFIED: | DATE: |
| OJAI POLICE | 805-656-1414 | WATCH COMMANDER | NOTIFY |
| VENTURA COUNTY FIRE | 805-371-1111 | STATION 21 | NOTIFY |

SECTION 7                                        Part 135 GH 4A   Part 133 GH 4L

Congested Area Plan

Operator:	Guardian Helicopters, Inc

67 D Street

Fillmore, California 93015

Evacuation 					Crowd Control 			Street Closure

No Evacuated Structures 			Operator 			Yes □ No

□Evacuated Structures 	           Contractor			Contractor

□Operator

LIST OF BUILDINGS THAT SHALL EITHER BE PARTIALLY OR ENTIRELY UNOCCUPIED PERSONS ALSO HOMES THAT WILL BE NOTIFIED OF ACTIVITY:

| BUILDING DESCRIPTION/ADDRESS | REMARKS | METHOD |
| --- | --- | --- |
| ALL HOMES WITHIN RED OPERATING AREA | VACANT DURING OPS | HAND DELIVER |
| SEE DIAGRAM = VACANT | VACANT DURING OPS | HAND DELIVER |

Narrative description of pick-up site, route, delivery site, and plan for ceasing operation if unauthorized persons enter operational area or real hazard occurs.

SITE INSPECTION NOTES

ONLY PERSONNEL ESSENTIAL TO COMPLETING THE JOB SAFELY WILL BE PERMITTED TO PARTICIPATE IN THE EVOLUTION. ONLY PERSONNEL PARTICIPATING IN THE SAFETY BRIEF WILL BE PERMITTED TO BE IN THE AREA OF OPERATIONS. ONLY AUTHORIZED ESSENTIAL PERSONAL WILL BE PERMITTED IN THE IMMEDIATE AREA OF OPERATIONS. IN THE EVENT AN UNAUTHORIZED PERSON ENTERS THE OPERATIONS AREA OR A POTENTIAL HAZARD IS IDENTIFIED THE PILOT WILL HOLD THE AIRCRAFT IN RELATIVE POSITION. THIS WILL BE ACHIEVED WITHOUT OVER FLYING THE GROUND CREW. ALL OPERATIONS WILL BE CEASED UNTIL THE PROBLEM IS RECTIFIED.  IN THE EVENT OF AN AIRCRAFT EMERGENCY, THE PILOT WILL RADIO GROUND PERSONNEL AND WILL ATTEMPT TO LAND IN THE EMERGENCY LANDING AREA.

DURING THE LIFT EVOLUTION THE AIRCRAFT AND GROUND CREW WILL ALL BE IN RADIO COMMUNICATIONS. IN THE EVENT OF LOSS COMMUNICATIONS, THE CREW WILL USE STANDARD HAND SIGNALS TO COMMUNICATE WITH THE PILOT. WHEN THE PILOT IS CLEAR HE WILL MOVE THE AIRCRAFT TO A SAFE LOCATION TO ALLOW THE COMMUNICATION PROBLEM TO BE RECTIFIED.

THE OPERATIONS AREA IS WITHIN THE SECURED AREA. ALL ESSENTIAL CREW WITHIN THE OPERATIONS AREA WILL NOT BE INVOLVED IN TAKING PICTURES OR ANY OTHER USE OF CELL PHONES.

PRIOR TO BEGINNING OPERATIONS, A TAILBOARD SAFETY BRIEF WILL BE CONDUCTED.

- HELICOPTER WILL LAND AT THE LZ TO CONNECT THE LONGLINE AND CONDUCT A TAILBOARD

- FUELING WILL BE CONDUCTED AT THE LANDING ZONE

- EMERGENCY LANDING ZONE IS AT THE STAGING AREA AND ALONG THE FLIGHT PATH

- LINE PULL WILL BEGIN AT THE START POINT NEAR THE LZ STAGING AREA

- CREEK ROAD AND CLARK RANCH ROAD WILL BE CLOSED INTERMITTENTLY

- EVACUATION NOT REQUIRED

- WATER TRUCK REQUIRED AT LZ STAGING AREA

- THE CONTRACTOR WILL TRIM TREES IF REQUIRED

4-3-2020

DATE OF SUBMISSION				COMPANY OFFICIAL / TITLE