SECTION 7                                        Part 135 GH 4A   Part 133 GH 4L

NON Congested Area Plan

Operator:	Guardian Helicopters, Inc

67 D STREET

FILLMORE , CA 93015

Phone (818) 442-9904

FAX   (818) 442-9901

PROJECT: 	EDISON POLE REPLACEMENT	WORK ORDER #: 410167262

ADDRESS:  1960 CATE MESA RD          	:  CARPENTARIA    CA

LZ STAGING AREA LATITUDE 34°24'23.43"N LONGITUDE 119°28'26.27"W

POLE LATITUDE 34°24'17.99"N LONGITUDE 119°28'52.17"W45 CL 1 COMP 1900 LBS

CONTRACTOR’S NAME: ISAAC ZAVALA  PHONE#: 805-760-2119

PILOT’S NAME(S):

1.  JOHN OLSON 			CERT #: 2769166

2. BUCKEY MACKAY	 	CERT #: 2778051

3. MICHAEL KLINK 		CERT#: 3018828

4. IAN GOODALE			CERT# :3741973

5. ROSS GOULET 			CERT# : 3396777

TYPE OF AIRCRAFT:

BELL UH-1H	 N# N711GH	A/C CATEGORY: RESTRICTED

BELL UH-1H	 N# N777GH	A/C CATEGORY: RESTRICTED

BELL 407	 N# N407GH	A/C CATEGORY: STANDARD

BELL 407	 N# N447GH 	A/C CATEGORY: STANDARD

BELL 205A1	 N# N216GH	A/C CATEGORY: STANDARD

AS350 B2	 N# N498PT	A/C CATEGORY: STANDARD

AS350 B3	 N# N718GH	A/C CATEGORY: STANDARD

AS350 B3	 N# N215GH	A/C CATEGORY: STANDARD

TYPE OF ITEM LIFTING: POLE       AERODYNAMIC   NON- AERODYNAMIC

IF SAFETY DICTATES AND OR THE CUSTOMER REQUESTS ADDITIONAL LIFTS THE LIFTS WILL BE CONDUCTED REMAINING WITHIN THE APPROVED TIME PARAMETERS.

TYPE OF LIFTS  POLES POLE REPLACEMENT

MAX SINGLE – LIFT WEIGHT:	1900  LBS	CLASS OF LOAD:	EXTERNAL 	CL B

NO. OF LIFTS: 4	LONG LINE:	 	(NO)	XXX	(YES)	LESS THEN 100 FT 	(LENGTH)

OPERATIONAL ALTITUDE: < 250 AGL   STREET CLOSING:  	  (N0)	XX	(YES)

DATE AND TIMES OPERATION WILL BEGIN AND TERMINATE:

| FLIGHT DATES | TIME BEGIN | TIME END | TODAY’S DATE |
| --- | --- | --- | --- |
| 6-27-2020 | SUNRISE | SUNSET | 6-24-2020 |
| AGENCY NAME: | PHONE#: | PERSON NOTIFIED: | DATE: |
| SANTA BARBARA COUNTY FIRE | (805) 965-5254 | STATION 4 | NOTIFY |
| SANTA BARBARA COUNTY SHERIFF | (805) 681-4100 | DISPATCH | NOTIFY |

NON- Congested Area Plan (Continued)

Evacuation 					Crowd Control 			Street Closure

□ No Evacuated Structures 			Operator 			Yes □ No

Evacuated Structures       		Contractor			Contractor

□Operator

LIST OF BUILDINGS THAT SHALL EITHER BE PARTIALLY OR ENTIRELY UNOCCUPIED PERSONS ALSO HOMES THAT WILL BE NOTIFIED OF ACTIVITY:

| BUILDING DESCRIPTION/ADDRESS | REMARKS | METHOD |
| --- | --- | --- |
| THREE BUILDINGS ON SCHOOL PROPERTY | VACANT DURING OPS | HAND DELIVER |
| ALL HOMES WITHIN OPERATING AREA | VACANT DURING OPS | HAND DELIVER |
| SEE DIAGRAM = V | VACANT DURING OPS | HAND DELIVER |

Narrative description of pick-up site, route, delivery site, and plan for ceasing operation if unauthorized persons enter operational area or real hazard occurs.

SITE INSPECTION NOTES

ONLY PERSONNEL ESSENTIAL TO COMPLETING THE JOB SAFELY WILL BE PERMITTED TO PARTICIPATE IN THE EVOLUTION. ONLY PERSONNEL PARTICIPATING IN THE SAFETY BRIEF WILL BE PERMITTED TO BE IN THE AREA OF OPERATIONS. ONLY AUTHORIZED ESSENTIAL PERSONAL WILL BE PERMITTED IN THE IMMEDIATE AREA OF OPERATIONS. IN THE EVENT AN UNAUTHORIZED PERSON ENTERS THE OPERATIONS AREA OR A POTENTIAL HAZARD IS IDENTIFIED THE PILOT WILL HOLD THE AIRCRAFT IN RELATIVE POSITION. THIS WILL BE ACHIEVED WITHOUT OVER FLYING THE GROUND CREW. ALL OPERATIONS WILL BE CEASED UNTIL THE PROBLEM IS RECTIFIED.  IN THE EVENT OF AN AIRCRAFT EMERGENCY, THE PILOT WILL RADIO GROUND PERSONNEL AND WILL ATTEMPT TO LAND IN THE EMERGENCY LANDING AREA.

DURING THE LIFT EVOLUTION THE AIRCRAFT AND GROUND CREW WILL ALL BE IN RADIO COMMUNICATIONS. IN THE EVENT OF LOSS COMMUNICATIONS THE CREW WILL USE STANDARD HAND SIGNALS TO COMMUNICATE WITH THE PILOT. WHEN THE PILOT IS CLEAR HE WILL MOVE THE AIRCRAFT TO A SAFE LOCATION TO ALLOW THE COMMUNICATION PROBLEM TO BE RECTIFIED.

THE OPERATIONS AREA IS WITHIN THE SECURED AREA. ALL ESSENTIAL CREW WITHIN THE OPERATIONS AREA WILL NOT BE INVOLVED IN TAKING PICTURES OR ANY OTHER USE OF CELL PHONES.

A TAILBOARD SAFETY MEETING WILL BE HELD AT THE ASSIGNED AREA PRIOR TO BEGINNING HELICOPTER OPERATIONS

1. HELICOPTER WILL AT THE LANDING ZONE OFF OF CATE MESA ROAD IN THE BASEBALL FIELD

2. FUELING WILL ONLY BE CONDUCTED AT THE LZ STAGING AREA

3 THE POLES WILL BE STAGED OFF OF CATE MESA ROAD TO THE NORTHWEST OF THE LANDING ZONE

6. EMERGENCY LANDING ZONE IS AT THE STAGING AREA AND ALONG THE FLIGHT PATH

7. THREE BUILDINGS ON THE CAMPUS WILL BE EVACUATED DURING THE OPERATIONS

8. WATER TRUCK IS REQUIRED AT LZ

9.  CATE MESA ROAD WILL  EXPERIENCE INTERMITTENT CLOSURES DURING OPERATION SEE DIAGRAM

10. TREE TRIMMER IS REQUIRED

OPERATIONS AREA DIAGRAM

LEGEND

| SITE PLAN SYMBOLS: EVACUATION FLIGHT PATH |  |  |
| --- | --- | --- |
| LANDING AREA = (LZ) | STREET BLOCKED = XXXX | TOP TWO FLOORS = UBT2 |
| ATTACH AREA = (A) | RELEASE AREA = (R) |  |
| FLIGHT PATH = >>> | EMERGENCY LAND = (ELZ) |  |
| UNOCCUPIED BUILDING = (UBA) | UNOCCUPIED BUILDING TOP FLOOR = (UBT1) |  |

The Helicopter will fly into the operating area from the north, maintaining VFR conditions at a safe operating altitude. Once the aircraft lands at the approved landing zone the crew will conduct a tailboard Safety brief. The following is a narrative of the operations;

- If required the pilot will fly equipment / tools to the lineman at the pole site.

- When the lineman crew are ready to receive the replacement pole the pilot will lift the pole vertically and fly along the approve flight path and set the pole.

- When the lineman crew are ready to remove the pole, the pilot will fly the empty hook to the pole site and lift the old pole and fly vertically to the Attached Release site.

All operations will be less then 250 ft AGL.

AREA SURVEY CONDUCTED BY :