SECTION 7                                        Part 135 GH 4A   Part 133 GH 4L

NON Congested Area Plan

Operator:	Guardian Helicopters, Inc

67 D STREET

FILLMORE , CA 93015

Phone (818) 442-9904

FAX   (818) 442-9901

PROJECT: 	PG&E POLE REPLACEMENT     	WORK ORDER #: 20-01401

ADDRESS:  MILL CREEK RD        	:  HEALDSBURG  CA

LZ STAGING AREA LATITUDE 38°35'36.18"N LONGITUDE 122°56'5.97"W

102029996  LATITUDE 38°35'18.00"N LONGITUDE 122°57'26.40"W 55 CL 1 2665 LBS

101963635 LATITUDE 38°35'17.77"N LONGITUDE 122°57'28.50"W55 CL 1 2665 LBS

CONTRACTOR’S NAME: TROY BARD         		    PHONE#: 707-291-2654

CONTRACTOR’S NAME: BRETT HUMPHREY           	    PHONE#: 707-708-1818

PILOT’S NAME(S): 	1.  JOHN OLSON 			CERT#:2769166

2. BUCKEY MACKAY	 	CERT#:2778051

3. MICHAEL KLINK 		CERT#:3018828

4. IAN GOODALE			CERT#:3741973

5. ROSS GOULET 			CERT#:339677

6.CHRIS CURRIER			CERT#:3093383

TYPE OF AIRCRAFT:

BELL UH-1H	 N#: N711GH	A/C CATEGORY: RESTRICTED

BELL UH-1H	 N#: N777GH	A/C CATEGORY: RESTRICTED

BELL 407	 N#: N407GH	A/C CATEGORY: STANDARD

BELL 407	 N#: N447GH	A/C CATEGORY: STANDARD

BELL 205A1	 N#: N216GH	A/C CATEGORY: STANDARD

AS350 B2	 N#: N498PT	A/C CATEGORY: STANDARD

AS350 B3	 N#: N718GH	A/C CATEGORY: STANDARD

AS350 B3	 N#: N215GH	A/C CATEGORY: STANDARD

TYPE OF ITEM LIFTING: POLE       AERODYNAMIC   NON- AERODYNAMIC

IF SAFETY DICTATES AND OR THE CUSTOMER REQUESTS ADDITIONAL LIFTS THE LIFTS WILL BE CONDUCTED REMAINING WITHIN THE APPROVED TIME PARAMETERS.

TYPE OF LIFTS 2 55 CL1 POLES

MAX SINGLE – LIFT WEIGHT:	2700  LBS	CLASS OF LOAD:	EXTERNAL 	CL B

NO. OF LIFTS: TBD	LONG LINE:	 	(NO)	XXX	(YES)	LESS THEN 200 FT 	(LENGTH)

OPERATIONAL ALTITUDE: < 250   AGL STREET CLOSING:  	  (N0)	XX	(YES)

DATE AND TIMES OPERATION WILL BEGIN AND TERMINATE:

| FLIGHT DATES | TIME BEGIN | TIME END | TODAY’S DATE |
| --- | --- | --- | --- |
| 7-29-2020 | SUNRISE | SUNSET | 7-8-2020 |
| AGENCY NAME: | PHONE#: | PERSON NOTIFIED: | DATE: |
| HEALDSBURG POLICE DEPT. | 707-431-3377 | DISPATCH | NOTIFY |
| HEALDSBURG FIRE DEPT. | 707-431-3360 | DISPATCH | NOTIFY |

NON- Congested Area Plan (Continued)

Evacuation 					Crowd Control 			Street Closure

□No Evacuated Structures 			Operator 			Yes □ No

Evacuated Structures         		Contractor			Contractor

□Operator

LIST OF BUILDINGS THAT SHALL EITHER BE PARTIALLY OR ENTIRELY UNOCCUPIED PERSONS ALSO HOMES THAT WILL BE NOTIFIED OF ACTIVITY:

| BUILDING DESCRIPTION/ADDRESS | REMARKS | METHOD |
| --- | --- | --- |
| ???? MILL CREEK RD SEE DIAGRAM | VACANT DURING OPS | HAND DELIVER |
| ALL HOMES WITHIN OPERATING AREA | VACANT DURING OPS | HAND DELIVER |
| SEE DIAGRAM = V | VACANT DURING OPS | HAND DELIVER |

Narrative description of pick-up site, route, delivery site, and plan for ceasing operation if unauthorized persons enter operational area or real hazard occurs.

SITE INSPECTION NOTES

ONLY PERSONNEL ESSENTIAL TO COMPLETING THE JOB SAFELY WILL BE PERMITTED TO PARTICIPATE IN THE EVOLUTION. ONLY PERSONNEL PARTICIPATING IN THE SAFETY BRIEF WILL BE PERMITTED TO BE IN THE AREA OF OPERATIONS. ONLY AUTHORIZED ESSENTIAL PERSONAL WILL BE PERMITTED IN THE IMMEDIATE AREA OF OPERATIONS. IN THE EVENT AN UNAUTHORIZED PERSON ENTERS THE OPERATIONS AREA OR A POTENTIAL HAZARD IS IDENTIFIED THE PILOT WILL HOLD THE AIRCRAFT IN RELATIVE POSITION. THIS WILL BE ACHIEVED WITHOUT OVER FLYING THE GROUND CREW. ALL OPERATIONS WILL BE CEASED UNTIL THE PROBLEM IS RECTIFIED.  IN THE EVENT OF AN AIRCRAFT EMERGENCY, THE PILOT WILL RADIO GROUND PERSONNEL AND WILL ATTEMPT TO LAND IN THE EMERGENCY LANDING AREA.

DURING THE LIFT EVOLUTION THE AIRCRAFT AND GROUND CREW WILL ALL BE IN RADIO COMMUNICATIONS. IN THE EVENT OF LOSS COMMUNICATIONS THE CREW WILL USE STANDARD HAND SIGNALS TO COMMUNICATE WITH THE PILOT. WHEN THE PILOT IS CLEAR HE WILL MOVE THE AIRCRAFT TO A SAFE LOCATION TO ALLOW THE COMMUNICATION PROBLEM TO BE RECTIFIED.

THE OPERATIONS AREA IS WITHIN THE SECURED AREA. ALL ESSENTIAL CREW WITHIN THE OPERATIONS AREA WILL NOT BE INVOLVED IN TAKING PICTURES OR ANY OTHER USE OF CELL PHONES.

A TAILBOARD SAFETY MEETING WILL BE HELD AT THE ASSIGNED AREA PRIOR TO BEGINNING HELICOPTER OPERATIONS

- HELICOPTER WILL AT THE LANDING ZONE AREA TO CONNECT LONGLINE AND CONDUCT A TAILBOARD SAFETY BRIEF

- WATER TRUCK REQUIRED

- PILOT WILL FLY LOW RECON PRIOR TO OPERATIONS TO VERIFY CLEAR FLIGHT PATH TO POLE SITE

- FUELING WILL ONLY BE CONDUCTED AT THE LZ STAGING AREA

- EMERGENCY LANDING ZONE IS AT THE STAGING AREA AND ALONG THE FLIGHT PATH

- ONE HOME REQUIRES EVACUATION SEE DIAGRAM

- MILL CREEK ROAD WILL EXPERIENCE INTERMITTENT CLOSURES

- TREE TRIMMING MAYBE REQUIRED PRIOR TO OPERATIONS

OPERATIONS AREA DIAGRAM

LEGEND

| SITE PLAN SYMBOLS: EVACUATION FLIGHT PATH |  |  |
| --- | --- | --- |
| LANDING AREA = (LZ) | STREET BLOCKED = XXXX | TOP TWO FLOORS = UBT2 |
| ATTACH AREA = (A) | RELEASE AREA = (R) |  |
| FLIGHT PATH = >>> | EMERGENCY LAND = (ELZ) |  |
| UNOCCUPIED BUILDING = (UBA) | UNOCCUPIED BUILDING TOP FLOOR = (UBT1) |  |

AREA SURVEY CONDUCTED BY :