SECTION 7                                        Part 135 GH 4A   Part 133 GH 4L

Congested Area Plan

Operator:	Guardian Helicopters, Inc

67 D STREET 					Phone (818) 442-9904

Fillmore CA. 93015 					FAX   (818) 442-9901

PROJECT: A/C UNITS 		     WORK ORDER #: Q2020-353

ADDRESS 44109 PACIFIC COMMONS BLVD    	:  FREMONT   CA

LZ STAGING AREA LATITUDE 37°29'50.47"N LONGITUDE 121°58'5.88"W

CONTACT AT SITE:	SHELDON MECHANICAL    DAN BOUTE	 PHONE#: 661-414-2024

CONTACT AT SITE:	PETE ALVAREZ FOREMAN PHONE#:661-505-2660

CONTRACTOR:    FAIRLIFTS AUSTIN QUATTROCCHI  PHONE#: 470-443-9800

PILOT’S NAME(S): 	1.  JOHN OLSON 			CERT #: 2769166

2. BUCKEY MACKAY	 	CERT #: 2778051

3. MICHAEL KLINK 		CERT#: 3018828

4. IAN GOODALE			CERT#: 3741973

5. ROSS GOULET 			CERT#: 3396777

TYPE OF AIRCRAFT:	BELL UH-1H	 N# N711GH	A/C CATEGORY: RESTRICTED

BELL UH-1H	 N# N777GH	A/C CATEGORY: RESTRICTED

BELL 407	 N# N407GH	A/C CATEGORY: STANDARD

BELL 205A1	 N# N216GH	A/C CATEGORY: STANDARD

AS350 B2	 N# N498PT	A/C CATEGORY: STANDARD

AS350 B3	 N# N718GH	A/C CATEGORY: STANDARD

AS350 B3	 N# N215GH	A/C CATEGORY: STANDARD

IF SAFETY DICTATES AND OR THE CUSTOMER REQUESTS ADDITIONAL LIFTS THE LIFTS WILL BE CONDUCTED REMAINING WITHIN THE APPROVED TIME PARAMETERS.

TYPE OF LIFTS: (19  ) 1975 LBS UNITS      LOAD:EXT CL B  TYPE OF ITEM LIFTING:  AERODYNAMIC   NON- AERODYNAMIC  LONG LINE: (NO)	XXX	(YES)	LESS THEN 100 FT 	(LENGTH)

OPERATIONAL ALTITUDE: < 250 AGL    STREET CLOSING:  XX	  (N0)		(YES)

DATE AND TIMES OPERATION WILL BEGIN AND TERMINATE:

| FLIGHT DATES | TIME BEGIN | TIME END | TODAY’S DATE |
| --- | --- | --- | --- |
| 7-22-2020 | SUNRISE | SUNSET | 7-14-2020 |
| AGENCY NAME: | PHONE#: | PERSON NOTIFIED: | DATE: |
| FREMONT POLICE DEPARTMENT | 510-790-6800 | DISPATCH | NOTIFIED |
| FREMONT FIRE DEPARTMENT | 510-494-4280 | DISPATCH | NOTIFIED |
| AIR TRAFFIC CONTROL | N/A | N/A | N/A |

Part 135 GH 4A   Part 133 GH 4L

Congested Area Plan

Evacuation 					Crowd Control 			Street Closure

□No Evacuated Structures 		 		Operator 			□Yes  No

Evacuated Structures 	      			 Contractor		Contractor

□Operator

LIST OF BUILDINGS THAT SHALL EITHER BE PARTIALLY OR ENTIRELY UNOCCUPIED PERSONS ALSO HOMES THAT WILL BE NOTIFIED OF ACTIVITY:

| BUILDING DESCRIPTION/ADDRESS | REMARKS | METHOD |
| --- | --- | --- |
| 44109 PACIFIC COMMONS BLVD AMAZON CENTER | VACANT DURING OPS | HAND DELIVER |
| ALL HOMES WITHIN RED OPERATING AREA | VACANT DURING OPS | HAND DELIVER |
| SEE DIAGRAM = VACANT | VACANT DURING OPS | HAND DELIVER |

Narrative description of pick-up site, route, delivery site, and plan for ceasing operation if unauthorized persons enter operational area or real hazard occurs.

SITE INSPECTION NOTES

ONLY PERSONNEL ESSENTIAL TO COMPLETING THE JOB SAFELY WILL BE PERMITTED TO PARTICIPATE IN THE EVOLUTION. ONLY PERSONNEL PARTICIPATING IN THE SAFETY BRIEF WILL BE PERMITTED TO BE IN THE AREA OF OPERATIONS. ONLY AUTHORIZED ESSENTIAL PERSONAL WILL BE PERMITTED IN THE IMMEDIATE AREA OF OPERATIONS. IN THE EVENT AN UNAUTHORIZED PERSON ENTERS THE OPERATIONS AREA OR A POTENTIAL

HAZARD IS IDENTIFIED THE PILOT WILL HOLD THE AIRCRAFT IN RELATIVE POSITION. THIS WILL BE ACHIEVED WITHOUT OVER FLYING THE GROUND CREW. ALL OPERATIONS WILL BE CEASED UNTIL THE PROBLEM IS RECTIFIED.  IN THE EVENT OF AN AIRCRAFT EMERGENCY, THE PILOT WILL RADIO GROUND PERSONNEL AND WILL ATTEMPT TO LAND IN THE EMERGENCY LANDING AREA.

DURING THE LIFT EVOLUTION THE AIRCRAFT AND GROUND CREW WILL ALL BE IN RADIO COMMUNICATIONS. IN THE EVENT OF LOSS COMMUNICATIONS, THE CREW WILL USE STANDARD HAND SIGNALS TO COMMUNICATE WITH THE PILOT. WHEN THE PILOT IS CLEAR HE WILL MOVE THE AIRCRAFT TO A SAFE LOCATION TO ALLOW THE COMMUNICATION PROBLEM TO BE RECTIFIED.

THE OPERATIONS AREA IS WITHIN THE SECURED AREA. ALL ESSENTIAL CREW WITHIN THE OPERATIONS AREA WILL NOT BE INVOLVED IN TAKING PICTURES OR ANY OTHER USE OF CELL PHONES.

PRIOR TO BEGINNING OPERATIONS, A TAILBOARD SAFETY BRIEF WILL BE CONDUCTED.

- TAILBOARD WILL BE CONDUCTED AT THE LANDING ZONE

- THE AIRCRAFT WILL LAND AT THE STAGING AREA TO CONNECT THE LONGLINE AND CONDUCT FUELING

- WATER TRUCK NOT REQUIRED

- EQUIPMENT WILL BE STAGED IN THE STAGING AREA TO THE SOUTH WEST OF THE BUILDING

- EMERGENCY LANDING ZONE IS AT THE STAGING AREA AND ALONG THE FLIGHT PATH

- PILOT AND GROUND CREW WILL BE IN RADIO COMMUNICATIONS

- SUBJECT BUILDING WILL BE VACANT DURING OPERATIONS.

- ALL ACCESS TO THE AMAZON CENTER TO THE OPERATIONS AREA WILL BE SECURED

- GATES WILL BE CLOSED TO THE FACILITY

- THE HELICOPTER WILL LAND AT THE STAGING AREA TO CONDUCT SAFETY BRIEF. ONCE THE UNITS ARE RIGGED AND EVERYONE IS IN PLACE THE PILOT WILL START THE OPERATION. THE UNITS WILL BE PLACED IN THE STAGING AREA IN A SEQUENTIAL ORDER. UNITS WILL BE STAGED ON THE SOUTHWEST  OF THE BUILDING. THE PILOT WILL LAND FOR FUEL AS NEEDED.

ONCE THE OPERATION IS COMPLETE THE PILOT WILL LAND AT THE LZ TO REMOVE THE LONGLINE. THE BUILDING CAN BE REOPENED.

LEGEND

| SITE PLAN SYMBOLS: EVACUATION FLIGHT PATH |  |  |
| --- | --- | --- |
| LANDING AREA = (LZ) | STREET BLOCKED = XXXX | TOP TWO FLOORS = UBT2 |
| ATTACH AREA = (A) | RELEASE AREA = (R) |  |
| FLIGHT PATH = >>> | EMERGENCY LAND = (ELZ) |  |
| UNOCCUPIED BUILDING = (UB) | UNOCCUPIED BUILDING TOP FLOOR = (UBT1) |  |

7-14-2020

DATE OF SUBMISSION				COMPANY OFFICIAL / TITLE

Jeffrey M. Teubner Flight Coordinator